Red protocol 字段定义

| JSON字段 |  | 字段定义 | UI显示 |
| --- | --- | --- | --- |
| model_no |  | 产品名称 | MODEL |
| sn |  | SN码 | SN |
| date |  | 生产日期 | Date Code |
| duration |  | 持续工作时间 | ALARM ON |
| sensor_status |  | 传感器状态 | SENSOR STATUS |
| battery_level |  | 电池格数 | 电池图标 |
| battery |  | 电池电压 | BATTERY |
| dust_level |  | 灰尘污染等级 | DUST CONTAMINATION |
| main_power_status |  | 主电状态 | 未发现UI |
| main_power_events | times | 主电断开次数 | MAIN POWER OFF |
| main_power_events | last_time | 距离上一次主电断开的时间 | MAIN POWER OFF |
| wrong_wiring_events | times | N线错误次数 | WRONG WIRING |
| wrong_wiring_events | last_time | 距离上次N线错误时间 | WRONG WIRING |
| wire_interconnect_events | times | 接驳报警次数 | WIRE INTERCONNECTION |
| wire_interconnect_events | last_time | 距离上次接驳报警时间 | WIRE INTERCONNECTION |
| Interconnect_events | times | RF报警次数 | INTERCONNECTION |
| Interconnect_events | last_time | 距离上次RF报警时间 | INTERCONNECTION |
| low_battery_events | times | 低电报警次数 | LOW BATTERY |
| low_battery_events | last_time | 距离上次低电报警时间 | LOW BATTERY |
| test_button_pressed | times | 测试按键被激活次数 | MANUAL TEST |
| test_button_pressed | last_time | 距离上次测试按键被激活的时间 | MANUAL TEST |
| times_alarm_deactivated | times | 报警器被关闭次数 | ALARM OFF |
| times_alarm_deactivated | last_time | 距离上次报警器被关闭的时间 | ALARM OFF |
| smoke_alarm | times | 烟雾报警次数 | OPTICAL SENSOR |
| smoke_alarm | last_time | 距离上一次烟雾报警的时间 | OPTICAL SENSOR |

| duration | duration >= 3650 | 提醒更换 |
| --- | --- | --- |
|  | 3285 <= duration < 3650 | 更换预警 |
|  | duration < 3285 | 运行正常 |
| battery_level | 4 | 电量高 |
|  | 3 | 电量中 |
|  | 2 | 电量低 |
| dust_level | 3 | 污染故障 |
|  | 2 | 污染预警 |
|  | 1 | 轻度污染 |
|  | 0 | 无污染 |
| sensor_status | 1 | 传感器故障 |
|  | 0 | 传感器正常 |
| main_power_status | 1 | 主电存在 |
|  | 0 | 主电不存在 |

JSON数据

- RFMDUAL (RGF-269)

- {

- "model_no": "RFMDUAL",

- "sn": "217055232",

- "date": "2026-04-10",

- "duration": 6,

- "sensor_status": 0,

- "battery_level": 4,

- "battery": 3.04,

- "dust_level": 1,

- "main_power_status": 0,

- "main_power_events": {

- "times": 1,

- "last_time": 1

- },

- "wrong_wiring_events": {

- "times": 2,

- "last_time": 2

- },

- "wire_interconnect_events": {

- "times": 3,

- "last_time": 3

- },

- "interconnect_events": {

- "times": 5,

- "last_time": 5

- },

- "low_battery_events": {

- "warning_beeps": 4,

- "last_beep": 4

- },

- "test_button_pressed": {

- "times": 6,

- "last_time": 6

- },

- "times_alarm_deactivated": {

- "times": 7,

- "last_time": 7

- },

- "smoke_alarm": {

- "times": 8,

- "last_time": 8

- }

- }

- R240P (AG-269)

{

"model_no": "R240P",

"sn": "217055232",

"date": "2026-04-10",

"duration": 6,

"sensor_status": 0,

"battery_level": 4,

"battery": 3.04,

"dust_level": 1,

"main_power_status": 0,

"main_power_events": {

"times": 1,

"last_time": 1

},

"wrong_wiring_events": {

"times": 2,

"last_time": 2

},

"wire_interconnect_events": {

"times": 3,

"last_time": 3

},

"interconnect_events": {

"times": 5,

"last_time": 5

},

"low_battery_events": {

"warning_beeps": 4,

"last_beep": 4

},

"test_button_pressed": {

"times": 6,

"last_time": 6

},

"times_alarm_deactivated": {

"times": 7,

"last_time": 7

},

"smoke_alarm": {

"times": 8,

"last_time": 8

}

}

- RSDUALP (AG-26-10Y)

{

"model_no": "RSDUALP",

"sn": "217055232",

"date": "2026-04-10",

"duration": 6,

"sensor_status": 0,

"battery_level": 4,

"battery": 3.04,

"dust_level": 1,

"main_power_status": 0,

"main_power_events": {

"times": 1,

"last_time": 1

},

"wrong_wiring_events": {

"times": 2,

"last_time": 2

},

"wire_interconnect_events": {

"times": 3,

"last_time": 3

},

"interconnect_events": {

"times": 5,

"last_time": 5

},

"low_battery_events": {

"warning_beeps": 4,

"last_beep": 4

},

"test_button_pressed": {

"times": 6,

"last_time": 6

},

"times_alarm_deactivated": {

"times": 7,

"last_time": 7

},

"smoke_alarm": {

"times": 8,

"last_time": 8

}

}

- R10RFP(AG-569-10YRF)

{

"model_no": "R10RFP",

"sn": "217055232",

"date": "2026-04-10",

"duration": 6,

"sensor_status": 0,

"battery_level": 4,

"battery": 3.04,

"dust_level": 1,

"interconnect_events": {

"times": 5,

"last_time": 5

},

"low_battery_events": {

"warning_beeps": 4,

"last_beep": 4

},

"test_button_pressed": {

"times": 6,

"last_time": 6

},

"times_alarm_deactivated": {

"times": 7,

"last_time": 7

},

"smoke_alarm": {

"times": 8,

"last_time": 8

}

}

5.Guardion Smoke Detector(HG-610RF)

{

"model_no": "Guardion Smoke Detector",

"sn": "217055232",

"date": "2026-04-10",

"duration": 6,

"sensor_status": 0,

"battery_level": 4,

"battery": 3.04,

"dust_level": 1,

"interconnect_events": {

"times": 5,

"last_time": 5

},

"low_battery_events": {

"warning_beeps": 4,

"last_beep": 4

},

"test_button_pressed": {

"times": 6,

"last_time": 6

},

"times_alarm_deactivated": {

"times": 7,

"last_time": 7

},

"smoke_alarm": {

"times": 8,

"last_time": 8

}

}

UI对应字段